This is how to make a controller for Real Race G2.

You can pick up an old joystick cord from a garage sale, or thrift shop for about a buck.

As for the gun, If you are lucky enough to find an old one that you want to sacrifice, your set, pull out the 5k Ohm pots, and replace with 500k Ohm ones that fit.

As for me I’ll use a piece of plywood, carved out in a rc-gun shape, and fit the pots in the right positions. And make up Wheel for steering, and cut out a piece of aluminum for the trigger - throttle. Any questions will be addressed thru alt.binaries.games.

Good luck. If anyone has a working copy Please post.

Parts

- 2	500k Ohm Pots

- 1 	Old Game Port cord, make sure it has all the pins

- 1	Momentary switch

- one piece of wire, light gauge 28-22 is fine

Game Port Pins

----------------------------------

(   1   2   3   4   5   6   7   8    )

( 9 10 11 12 13 14 15 16 )

-----------------------------

Game Port Wires Used

Yellow, Blue, Red, Green, Grey, Brown, Purple, Black

Steering (End Pot) Uses Pins 1,11,13 (Grey, Brown, Purple)

Steering (Centre Pot) pin 6 – Green

Throttle pin 3 – Black

Switch side 1 – Pin 2,10, (blue, Red)

Switch side 2 – Pin 4 (yellow)

Pins-

(These pins refer to pins on the 500k pots only, not game port pin numbers)

Steering Pot     (1) (2) (3)

Pin 2 (Centre) – Green wire

Pin 3 (End) – Grey, Brown, Purple (If steering is backwards, Use Pin 1 on Pot)

-(Solder another piece of wire here about 10cm(5inches), this lead will be connected to the other pot) (Red/extra) this is what I will refer to this wire as

Throttle Pot     (1) (2) (3)

Pin 2 (Centre) – Black

Pin 3 (End) – (Red/extra) – lead from steering pot Pin 3 (If throttle is backwards, Use pin 1 on Pot)

Momentary Switch   (1) (2) – refer to the two connections on switch

Pin 1 – Yellow

Pin 2 – Blue, Red